Title

: New Zealand’s hydrogeological unit dataset.

Date

: First Release: November 2019 (White et al. 2019)

: Current Release: May 2026

Abstract

The NZ_hydrogeological_unit_stacked_north_island_20260514.zip’ dataset consists of the following complementary files:

- ‘NZ_hydrogeological_unit_stacked_north_island_20260514.shp’ which consist of overlapping stacked polygons that represent hydrogeological units (i.e., aquifers, aquitards, aquicludes and basement) developed in a nationally-consistent manner.

- ‘HUM_data_dictionary_20260514.csv’ which summarises attributes, values and descriptions from the GIS dataset;

- ‘HUM_unit_list_20260514.csv’ which lists HUM unit names;

- ‘HUM_facies_list_20260514.csv’ which lists facies names;

- ‘HUM_facies_model_list_20260514.csv’ which lists current facies model and their definition criteria.

Each polygon is attributed with the following attributes (dataset attribute names in brackets):

- unique identification number (HUM_ID),

- unique name (HUM_name),

- hydrogeological unit type (HUM_type),

- geological era (HUM_era),

- geological age descriptor (HUM_age),

- lithological type (HUM_lith),

- lithological descriptor (HUM_rock),

- polygon-specific depositional facies name (facies),

- where applicable, hydrogeological system-specific reference to the representative volcanic facies model (volc_model),

- where applicable, hydrogeological system-specific reference to the representative sedimentary facies model (sed_model),

- polygon-specific outcrop indicator (outcrop),

- polygon-specific vertical sequence order, from younger to older unit (order),

- unique polygon id (poly_ID).

This release includes minor fixes and new polygons to fill the gap between the Clutha, Taieri and Waikato rivers polygons and the shoreline. This version supersedes the January 2026 version (White et al. 2019).

This dataset was initially prepared for the Ministry for the Environment (White et al. 2019) and is now developed by GNS as part of the Groundwater Strategic Science Investment Funds programme.

Release notes

Minor fixes: The outcrop attributes of multiple overlapping polygons were edited to ensure that only reflect the HUM unit shown on QMAP was attributed an outcrop value of 1 in the Reefton and Dunedin areas.

Minor modifications: HUM polygons were created to fill selected inland river polygons connected to the sea. There modifications applied to: the Waikato River between Hamilton and Tauranganui; the Clutha River between Roxburgh and Balclutha, and the Taieri River between Lake Waihola and Taieri Mouth.

License

: Use of the datasets is licensed under a Creative Commons Attribution 4.0 International (CC BY 4.0) Licence. You are free to share and adapt the datasets for any purpose provided you give appropriate credit, a link to the license, and note the changes that were made. For more details visit http://creativecommons.org/licenses/by/4.0/. Where the datasets are used in a figure, GNS Science requests attribution in the following manner: © GNS Science 2026.

Disclaimer

: In compiling the datasets, inferences and assumptions have been made about hydrogeological systems at a regional scale (i.e., 1:250,000). At the time of publishing, no validations using actual observations have been made (e.g., yield, aquifer properties, etc.), and the datasets do not include any information regarding the sustainability of a hydrogeological system (e.g., recharge/discharge areas, water balance, etc.). Experience and an appreciation of the limitations of the datasets is needed by persons using the datasets as an element in their decision making over access to and use of groundwater resources. In addition, the datasets should be treated with caution for detailed studies at map scales of less than 1:250,000.

The data user acknowledges that neither GNS Science nor any of its representatives has made or makes any representation or warranty, expressed or implied, as to the accuracy or completeness of the datasets or the accompanying report. GNS Science accepts no responsibility for any use or reliance on the datasets and shall not be liable to any person, on any ground, for any loss, damage or expense arising from such use or reliance.

If the data user makes the datasets available to any third party, the data user indemnifies and holds harmless GNS Science for any event arising from such use or supply.

Any party using or relying on the datasets will be regarded as having accepted the terms of this disclaimer.

Update frequency

: There is currently no scheduled update for this dataset. it is therefore recommended to include the current release date when using and referencing the dataset.

Resource constraints

: This dataset is provided under Creative Commons C BY 4.0.0 http://creativecommons.org/licenses/by/4.0/.

Where the dataset is used in a figure, it requires attribution in the following manner: © GNS Science 2026. Where reference to the dataset is to be included in a reference list please cite White et al., 2019 (Revised 2026) (see Credits for full reference).

Spatial resolution

: This dataset is provided with an indicative resolution of 1:250,000-scale. Details on uncertainty pertaining to scale, resolution and capture of the input dataset is detailed in Moreau et al., 2019 (see Credits for full reference).

Spatial Extent

: New Zealand, bounding box 'NZ': ('New Zealand', (166.509144322, -46.641235447, 178.517093541, -34.4506617165))

Temporal Extent

: Not applicable

Credits

: White PA, Moreau M, Mourot F, Rawlinson ZJ. 2019 (Revised 2026). New Zealand groundwater atlas:hydrogeological-unit map of New Zealand. Lower Hutt (NZ): GNS Science. [86] p. Consultancy Report 2019/144.

References

Bishop DG, Forsyth PJ, Turnbull IM, compilers. 1996. Dunedin [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 52 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 21).

Begg JG, Johnston MR, compilers. 2000. Wellington [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 1 book (64 p.), scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 10).

Cooper AF, Ireland TR. 2013. Cretaceous sedimentation and metamorphism of the western Alpine Schist protoliths associated with the Pounamu Ultramafic Belt, Westland, New Zealand, New Zealand Journal of Geology and Geophysics, 56:4,188-199, DOI: 10.1080/00288306.2013.809776

Cox SC, Barrell DJA, Suggate RP, compilers. 2007. Aoraki [map]. Lower Hutt (NZ): GNS Science. 1 map + 71 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 15).

Edbrooke SW, Brook FJ, compilers. 2009. Whangarei [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 68 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250 000 geological map; 2).

Edbrooke SW, compiler. 2001. Auckland [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 74 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 3).

Edbrooke SW (compiler) 2005: Geology of the Waikato area: scale 1:250,000. Lower Hutt: Institute of Geological & Nuclear Sciences. Institute of Geological & Nuclear Sciences 1:250,000 geological map 4. 68 p. + 1 folded map.

Environment Court. 2011. Decision No. NZEnvC 163 in regards of Proposed Waikato Regional Plan Variation 5 – Lake Taupo catchment. 48 p.

Forsyth PJ, Glassey PJ, Barrell DJA, compilers. 2001. Waitaki [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 64 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 19).

Forsyth PJ, Barrell DJA, Jongens R, Begg JG, compilers. 2008. Christchurch [map]. Lower Hutt (NZ): GNS Science. 1 map +67 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 16).

Heron DW. 2014. Geological map of New Zealand 1:250,000. Lower Hutt (NZ): GNS Science. 1 CD. (GNS Science geological map; 1).

Isaac MJ. 1996. Kaitaia [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 44 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 1).

Lee JM, Begg JG, compilers. 2002. Wairarapa [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 66 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 11).

Lee JM, Browne GH, Begg JG, compilers. 2011. Hawke's Bay [map]. Lower Hutt (NZ): GNS Science. 1 map + 93 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 8).

Lee JM, Bland KJ, Townsend DB, Kamp PJJ. (compilers) 2011. Geology of the Hawkes Bay. area. Institute of Geological and Nuclear Sciences geological map 8: scale 1:250,000. Lower Hutt.

Leonard, GS, Begg JG, Wilson CJN, 2010. Institute of Geological and Nuclear Sciences geological map 5 Geology of the Rotorua area: scale 1:250,000. Lower Hutt.

Leonard GS, Begg JG, Wilson CJN, compilers. 2010. Rotorua [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 102 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 5).

Leask WL. 1993. Brunner Coal Measures at Golden Bay, Nelson: an Eocene fluvial-estuarine deposit. New Zealand Journal of Geology and Geophysics 36, 37-50.

Mazengarb C, Speden IG, compilers. 2000. Raukumara [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 60 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 6).

Moreau M, White PA, Mourot F, Rawlinson Z, Tschritter C, Cameron SC, Westerhoff R. 2019. Classification of New Zealand hydrogeological systems. Lower Hutt (NZ): GNS Science. [28] p. (GNS Science report; 2018/35). doi:10.21420/42QW-MC74.

Nathan S, Rattenbury MS, Suggate RP, compilers. 2002. Greymouth [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 58 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 12).

Price RC, Mortimer N, Smith IEM, Maas R. 2015. Whole-rock geochemical reference data for Torlesse and Waipapa terranes, North Island, New Zealand, New Zealand, Journal of Geology and Geophysics, 58:3, 213-228, DOI: 10.1080/00288306.2015.1026832

Raiber M, White P, Daughney CJ, Bainbridge SE. 2012. Three-dimensional geological modelling and multivariate statistical analysis of water chemistry data to analyse and visualise aquifer structure and groundwater composition in the Wairau Plain, Marlborough District, New Zealand, Journal of Hydrology s 436-437:13-34.    DOI: 10.1016/j.jhydrol.2012.01.045

Rawlinson ZJ, Begg J. 2014. Hydrogeology of the Upper Manawatu and Mangatainoka catchments, Tararua, GNS Science Consultancy Report 2014/127. 48 p

Rattenbury MS, Cooper RA, Johnston MR, compilers. 1998. Nelson [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 67 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 9).

Rattenbury MS, Townsend DB, Johnston MR, compilers. 2006. Kaikoura [map]. Lower Hutt (NZ): GNS Science. 1 map + 70 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 13).

Rattenbury MS, Jongens R, Cox SC, compilers. 2010. Haast [map]. Lower Hutt (NZ): GNS Science. 1 map + 58 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 14).

Seebeck H, Strogen DP, Nicol A, Hines BR, Bland KJ. 2024. A tectonic reconstruction model for Aotearoa-New Zealand from the mid-Late Cretaceous to the present day, New Zealand Journal of Geology and Geophysics, 67:4,527-550, DOI: 10.1080/00288306.2023.2239175

Todd DK, Mays LW. 2005. Groundwater hydrology. 3rd ed. Hoboken (NJ): Wiley. 636 p.

Townsend DB, Vonk A, Kamp PJJ, compilers. 2008. Taranaki [map]. Lower Hutt (NZ): GNS Science. 1 map + 77 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 7).

Turnbull IM, compiler. 2000. Wakatipu [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 72 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 18).

Turnbull IM, Allibone AH. 2003. Murihiku [map]. Lower Hutt (NZ): Institute of Geological & Nuclear Sciences. 1 map + 74 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 20).

Turnbull IM, Allibone AH, Jongens R, compilers. 2010. Fiordland [map]. Lower Hutt (NZ): GNS Science. 1 map + 97 p., scale 1:250,000. (Institute of Geological & Nuclear Sciences 1:250,000 geological map; 17).

White PA, Moreau M, Mourot F, Rawlinson ZJ. 2019 (Revised 2022). New Zealand groundwater atlas: hydrogeological-unit map of New Zealand. Lower Hutt (NZ): GNS Science. [86] p. Consultancy Report 2019/144.

White PA, Moreau M, Reeves RR, Rae A, Tschritter C.2018. Evolution of geothermal-groundwater circulation systems and geology in the Quaternary Taupo Volcanic Zone: example of the Upper Waikato catchment, New Zealand. 40th Geothermal Workshop,14 – 16 November, Wairakei.

White PA, Raiber M, Tschritter C. 2018 Geological model and water budget of the Hauraki Plains, Waikato region. GNS Science consultancy report 2015/232. 63 p.

White PA, Tschritter C, Moreau MF. 2016 Geological model of the Upper Waikato River catchment for application to groundwater resources. GNS Science consultancy report 2015/199. 41 p.

Wilson CJN, Houghton BF, Lloyd EF. 1986. Volcanic history and evolution of the Maroa-Taupo area, Central North Island. In Smith, I. E. M. (Ed) Late Cenozoic Volcanism in New Zealand. The Royal Society of New Zealand. Bulletin 23. 194-223.